ZBX00030301

P/N : MUL-53221-06

PROGRAMMING GUIDE

PROGRAMMING
GUIDE

for

###### BARCODE

###### SCANNERS

The guide can be used as keyboard emulation, RS- 232C serial interface, and CMOS serial interface and wand emulation.

IMPORTANT NOTICE

This is a general guide for varies scanners, and not all functions will perform in every scanners.  Other than specified in this guide, for any special functions or specifications, please contact your dealer for details.

Every effort is made to ensure the accuracy of our product information; however, we accept no responsibility for errors or omissions including, but not limited to, the implied warranties of merchantability and fitness for a particular purpose.  We shall not be liable for errors contained herein or for incidental consequential damages in connection with the furnishing, performance, or use of this material.

Specification or version may be subject to change without notice. The actual specification and version are based on the product delivered.

PRINTED IN DECEMBER 2001

TABLE OF CONTENTS

Introduction	1

Default Parameters	2

Program Procedure	5

System Setting	6

General Configuration

Scanning Mode Selection	9

Inter- Message Delay	10

Inter- Character Delay	11

Message/ Block Mode Selection	11

Beeper Tone Selection	12

Interface Configuration

RS-232C Serial Communication Parameters Setting

Handshaking Protocol	15

ACK/ NAK Response Time Setting	16

Baud Rate	16

Data Bit	17

Stop Bit	17

Parity Setting	17

Message Terminator	18

Keyboard Emulation Parameters Setting	19

Keyboard Type Selection	19

Language Selection	21

Message Terminator	22

Function Key Emulation	23

Capital Lock Setting	23

Wand Emulation Parameters Setting	24

Emulation Data Output Selection	25

Wand Emulation Narrow/Wide Ratio	25

Cursor Pad Work at Numlock	25

USB Interface Parameters Setting	26

Keyboard Type	26

Message Terminator	26

The Symbologies

Reading Code Selection	28

Code 39 Parameters Setting	30

Interleaved 2 Of 5 Parameters Setting	31

i

Chinese Post Code Parameters Setting	32

UPC/ EAN/ JAN Parameters Setting	33

Coda bar/ Monarch Parameters Setting	37

Code 128 Parameters Setting	37

MATRIX 25 Parameters setting	38

MSI/Plessy parameters setting	39

Italian Pharmacy Parameters Setting	40

Barcode Length Setting	41

ISBN/ ISSN Conversion	42

Data Editing

Header and Trailer	43

Barcode Identifier Code Selection	44

Barcode Identifier Code Setting	45

Truncate Header/Trailer Character	47

Appendixes

Appendix A

Code 39 Full ASCII Code Table	49

Appendix B

Code 39 Full ASCII Bar Code Table	52

Appendix C

Barcode Samples	62

Appendix D

Quick Settings	63

ii

1. INTRODUCTION

Scanning a series of programming bar code labels can configure the series scanners.  This allows decoding options and interface protocols to be tailored to a specific application.  The configuration is stored in non-volatile memory and will not be lost by removing power from the scanner.

The scanner must be properly powered before programming.  For RS-232C type scanners, an external power adapter must be used to supply DC power to the scanner.  If a keyboard emulation type scanner is used with an IBM PC/XT/AT, PS/2 or any fully compatible computers, power will be drawn from the keyboard port.  No external power adapter is required.  If keyboard emulation type scanner is used with any other non IBM PC compatible computers, an external power adapter may be needed.

During the programming mode, the laser scanner will acknowledge a good and valid reading with a short beep.  It will give long beeps for either an invalid or bad reading.

2. PROGRAMMING OPTIONS

Programmable options are divided into four groups.  The first group includes the options that show the general behavior of the laser scanner.  The second group of options governs the operation of RS-232C type serial ports.  The third group selects the keyboard type that the keyboard emulation type will be emulated.  The last group sets the decoding parameters for each barcode symbology.

1

3. DEFAULT PARAMETERS

This table gives the default settings of all the programmable parameters.  The default settings will be restored whenever the "Reset" programming label is scanned and the laser scanner is in programming mode.

DEFAULT VALUES OF OPERATING PARAMETERS

|  | Function | Default Values |
| --- | --- | --- |
|  | Scanning Mode Selection | Trigger mode |
|  | Header and trailer | None |
|  | Inter-Message delay | Normal |
|  | Inter-Character delay | Normal |
|  | Message/Block mode selection | Message |
|  | Send command in block mode communication | Disable |
|  | Good read beeper tone selection | Medium |
|  | Code identifier transmitting | Disable |

## PREDEFINED BARCODE IDENTIFIERS*

|  | Code 39 barcode identifier code | M |
| --- | --- | --- |
|  | ITF 2 of 5 barcode identifier code | I |
|  | Chinese post code identifier code | H |
|  | UPC-E barcode identifier code | E |
|  | UPC-A barcode identifier code | A |
|  | EAN-13 barcode identifier code | F |
|  | EAN-8 barcode identifier code | FF |
|  | Codabar barcode identifier code | N |
|  | Code 128 barcode identifier code | K |
|  | Code 93 barcode identifier code | L |
|  | MSI barcode identifier code | P |
|  | MATRIX 25 barcode identifier code | G |

*

2

DEFAULT VALUES OF KEYBOARD EMULATION PARAMETERS SETTING

|  | Function | Default Values |
| --- | --- | --- |
|  | Keyboard type selection | IBM PC/AT USA |
|  | Message terminator | Enter/ carriage Return |

DEFAULT VALUES OF RS-232C SERIAL COMMUNICATION PARAMETERS

|  | Function | Default Values |
| --- | --- | --- |
|  | Handshaking protocol | None |
|  | ACK/NAK response time setting | 300 msec |
|  | Baud rate | 9600 |
|  | Data bit | 8 |
|  | Stop bit | 1 |
|  | Parity | Mark |
|  | Message terminator selection | CR/LF |

DEFAULT VALUES OF WAND EMULATION PARAMETERS

|  | Function | Default Values |
| --- | --- | --- |
| ※ | Wand emulation speed | Normal |
| ※ | Wand emulation output | Black = High |

Note: For wand emulation, the configuration is only effective for the items with asterisk (※).

DEFAULT VALUES OF USB EMULATION PARAMETERS

|  | Function | Default Values |
| --- | --- | --- |
| ※ | Keyboard Type | US Keyboard |
| ※ | Message Terminator | Enter |

3

# DEFAULT VALUES OF DECODING PARAMETERS

| Function | Code | Default Value |
| --- | --- | --- |
|  | Code 39 | Enable |
|  | ITF 2 of 5 | Enable |
|  | Chinese Post Code | Disable |
|  | UPC/EAN/JAN | Enable |
|  | Coda bar | Enable |
| Reading codes | ※ MSI | Disable |
| Selection | Code 128 | Enable |
|  | Code 93 | Enable |
|  | ※ ITAT | Disable |
|  | ※ EAN-128 | Disable |
|  | ※ MATRIX 25 | Disable |
|  | ※ Italian Pharmacy | Disable |
|  | ISSN/ ISBN | Disable |
|  | Codes | Standard |
| Code 39 | Start/stop characters | Not transmitting |
|  | Check digit | Disabled |
|  | Concatenation | Off |
| Interleaved | Length | 6-32 digits |
| 2 of 5 | Check digit | Disable |
| Chinese Post | Length | 10~16 digits |
| Code | Check digit | Transmit |
|  | Format | All |
|  | Addendum | Disable |
|  | UPC-E=UPC-A | Disabled |
| UPC/EAN/JAN | UPC-A leading digit | Transmit |
|  | UPC-A check digit | Transmit |
|  | UPC-E leading digit | Transmit |
|  | UPC-E check digit | Transmit |
|  | Type | Standard |
| Coda bar | Start/stop characters | A, B, C, D |
|  | Length | 6~32 digits |
| Code 128 | FNC 2 append | Disable |
|  | Check digit | Disable |
| MSI | Length | Variable |
|  | Check digit | Transmit |
| Italian Pharmacy | Transmit "A" Character | Not transmitting |
| MATRIX 25 | Length | Fix 10 digits |
|  | Check digit | Disable |

Note: The configuration of the items with asterisk

(※) is effective when being appointed in advance.

4

4. PROGRAM PROCEDURE USING BARCODE MENUS

NO

NO

YES

YES

5

##### SYSTEM SETTING

6

| Start of Configuration |  |
| --- | --- |
| RESET | - The reading of the "RESET” label turns all the parameters back to default values. - When you intend to turn your scanner back to default parameter, please scans the "Start of configuration" label first, then scan "RESET" label and finally scan the "End of configuration" label. |
| ABORT RS-232C PC/AT USB WAND EMULATION SHOW VERSION | - The reading of the "ABORT" label discards all the parameters read prior to the "End of configuration". - The scanner remains in the last interface mode when the scanner is reset. The label below should be scanned if the scanner is configured the first time. - The reading of the “SHOW VERSION” label will be show firmware version. |
|  | End of Configuration |

7

## GENERAL CONFIGURATION

8

| Start of Configuration |  |
| --- | --- |

## SCANNING MODE SELECTION (for laser scanner)

For series laser scanners, there are 3 scanning modes to suit your application requirements.

| Trigger Mode Pulse Mode | The scanner becomes inactive as soon as the data is transmitted. It must be triggered to become active again. The scanner will light up when press the pulse mode trigger switch once. And, the scanner will turn off for next pressing. |
| --- | --- |

## SCANNING MODE SELECTION (For CCD scanner)

| The scanner becomes inactive as soon as the data is transmitted. It must be triggered to become active again. In auto scan mode, the scanner is still active after the data is transmitted, but the successive transmission of the same bar code is not allowed when the trigger switch is pressed again. This scanner will light up when press the scanner trigger switch once. And, the scanner will turn off for next pressing. This mode is similar to Auto scan mode, but double reading for the same barcode is prohibited if the scanner switch is pressed. |  |  | Trigger mode Auto scan mode Alternate mode Repeat mode |  |
| --- | --- | --- | --- | --- |
|  |  | End of Configuration |  |  |

9

| Start of Configuration |  |
| --- | --- |

##### DATA REDUNDANT CHECK

| The option allows you to set decoder data redundant check. | Enable Disable |
| --- | --- |

## INTER-MESSAGE DELAY

These series of scanners allow you to add a delay between two consecutive messages.  This delay will be added before each data transmission.

None 100 msec 500 msec 1 Second

10

## INTER-CHARACTER DELAY

This option governs delay time between consecutive characters.  Scanning the following labels can alter the delay time.

None 10 msec 20 msec 50 msec

## MESSAGE/BLOCK MODE SELECTION

This option allows you to treat scanned data as either an independent message or a block message.  In the message mode, the data scanned will be transmitted immediately.  In block mode, the data scanned will be appended to the message buffer if the scanner is programmed in block mode.  A block of message will only be transmitted after a “Send” command is entered.  This mode is only available when the scanner is working with code 39 labels.  You are free to choose any character as the “ Send” command.

| Message Block |  |  |
| --- | --- | --- |
|  | End of Configuration |  |

11

| Start of Configuration |  |
| --- | --- |

SEND COMMAND IN BLOCK MODE COMMUNICATION

You can use this option to set your own “Send” command used in block mode communication.

Enable Disable Store Set

GOOD READ BEEPER TONE SELECTION

You can use this option to set frequency and / or duration of the buzzer after successful reading.

Medium Low High Disable

12

SOUND DURATION

long(120 ms) Medium(50 ms) Short(20 ms) Very short(5 ms)

*For Alpha-70 and SC-2070 Series only Medium and disable setting available, it’s hardware beeper control.

|  | End of Configuration |
| --- | --- |

13

INTERFACE CONFIGURATION

14

| Start of Configuration |  |
| --- | --- |

1.  RS-232C SERIAL COMMUNICATION PARAMETERS SETTING

The RS-232C scanner supports four handshaking protocols. With these options of communication protocol, you can tailor the scanner to meet the requirement of most systems

HANDSHAKING PROTOCOL

None RTS/CTS ACK/NAK Xon/Xoff

ACK/NAK RESPONSE TIME SETTING

| 300 msec 2 sec 500 msec |  |  |
| --- | --- | --- |
|  |  | End of Configuration |

15

| Start of Configuration |  |
| --- | --- |

ACK/NAK RESPONSE TIME SETTING (Cont’d)

3 sec 1 sec 5 sec

BAUD RATE

19200 9600 4800 2400 1200 600

16

DATA BIT

7 8

STOP BIT

1 2

PARITY

Even Odd Mark Space None

|  | End of Configuration |
| --- | --- |

17

| Start of Configuration |  |
| --- | --- |

MESSAGE TERMINATOR (FOR RS-232C TYPE ONLY)

None CR/LF CR LF H Tab STX/ETX EOT

18

2. KEYBOARD EMULATION PARAMETERS SETTING

KEYBOARD TYPE SELECTION

The keyboard emulation scanners can emulate a number of personal computers keyboard and a number of terminal keyboard. Keyboard emulation is activated whenever you have selected the type of keyboard for which the scanner is going to emulate. Choose the appropriate type of keyboard emulation by scanning the labels under the following labels.

| IBM AT PS/2 30-80 IBM 5550 IBM 5295 Terminal IBM XT IBM 5530-SC IBM 5530-ZC |  |  |
| --- | --- | --- |
|  | End of Configuration |  |

19

| Start of Configuration |  |
| --- | --- |

KEYBOARD TYPE SELECTION (Cont’d)

NEC 9801 IBM 3196 Terminal APPLE MAC II(※) IBM 3477/3472 Terminal PS2/30/56 IBM 3477 Terminal (Without break code) NEC 5200(※)

Note: The configuration of the items with asterisk (※) is optional.

20

KEYBOARD LANGUAGE SELECTION

USA UK Germany French Spanish Italian Swiss Swedish

|  | End of Configuration |
| --- | --- |

21

| Start of Configuration |  |
| --- | --- |

MESSAGE TERMINATOR (FOR KEYBOARD WEDGE USE)

None Return /Enter Hor. TAB Execute

KEYBOARD TYPE SELECTION

Scan Code Mode Alt mode

BREAK CODE ON/ OFF SETTING (FOR IBM Terminals 31xx, 34xx, 37xx USE)

To select the interface for these IBM terminals, read the correct key transmission code.

ON OFF

22

FUNCTION KEY ACTIVE ON/ OFF (FOR IBM AT USE)

Function keys can be concatenated with input data as header and/or trailer. See table on page 40.

ON OFF

CAPITAL LOCK ON/ OFF

Select the suitable code to match your keyboard caps lock status.

ON OFF

### Function key emulation (only for PC/AT) ### Numlock on/off OFF ON

|  | End of Configuration |
| --- | --- |

23

| Start of Configuration |  |
| --- | --- |

00H~1FH ASCII Code defined

Alt-mode code

## Crtl+code

## 3. WAND EMULATION PARAMETERS SETTING

## EMULATION SPEED SELECTION

Low Medium Normal High Higher

24

## EMULATION DATA OUTPUT SELECTION

The decoded data output logic level can be set to befit the external decoder.

Black = High Black = Low

## WAND EMULATION NARROW/WIDE RATIO

1:2 1:3

CURSOR PAD WORK AT NUMLOCK

ON OFF

|  | End of Configuration |
| --- | --- |

25

| Start of Configuration |  |
| --- | --- |

4. USB INTERFACE PARAMETERS SETTING

The USB mode is effectively a keyboard emulator that works with hosts that USB-compatible operating system and USB ports.  USB compatible operating systems are Windows 98, Windows NT 5.0 and later, no additional software is needed since the USB driver support is built into this operating system.

KEYBOARD TYPE

US Keyboard International Keyboard

Message Terminator

None Enter H Tab

|  | End of Configuration |
| --- | --- |

26

THE SYMBOLOGIES

27

| Start of Configuration |  |
| --- | --- |

READING CODE SELECTION

Code 39 Enable Code 39 Disable Coda bar Enable Coda bar Disable UPC/ EAN/ JAN Enable UPC/ EAN/ JAN Disable ITF 2 of 5 Enable ITF 2 of 5 Disable Chinese Post Code Enable Chinese Post Code Disable Code 128 Enable

28

| Code 128 Disable MSI Enable MSI Disable Code 93 Enable Code 93 Disable IATA Enable IATA Disable EAN- 128 Enable EAN-128 Disable MATRIX 25 Enable MATRIX Disable |  |
| --- | --- |
|  | End of Configuration |

29

| Start of Configuration |  |
| --- | --- |

READING CODE SELECTION (Cont’d)

Italian Pharmacy Enable Italian Pharmacy Disable

CODE 39 PARAMETERS SETTING

CHARACTER SET Standard Code 39 Full ASCII Code 39 START/STOP CHARACTER TRANSMISSION Yes No CHECK DIGIT Calculate and Transmit Calculate but not Transmit

30

NO CONCATENATION Enable Disable

INTERLEAVED 2 OF 5 PARAMENTERS SETTING

Examples: Felting length 4 to 8 digits

LENTGTH

| Scan:. Start of configuration Min 0 4 set | max 0 8 set End of configuration | MAX Min Set |
| --- | --- | --- |

|  | End of Configuration |
| --- | --- |

31

| Start of Configuration |  |
| --- | --- |

CHECK DIGIT

NO Calculate and Transmit Calculate but not Transmit

CHINESE POST CODE PARAMETERS SETTING

LENGTH

| MAX MIN Set | Scan:. Start of configuration Min 0 4 set | max 0 8 set End of configuration |
| --- | --- | --- |

32

CHECK DIGIT

NO Calculate and Transmit Calculate but not Transmit

UPC/EAN/JAN PARAMETERS SETTING

FORMAT

| All EAN-8 or EAN-13 UPC-A and EAN-13 UPC-A and UPC-E UPC-A UPC-E |  |
| --- | --- |
|  | End of Configuration |

33

| Start of Configuration |  |
| --- | --- |

UPC/EAN/JAN PARAMETERS SETTING (Cont’d)

EAN-13 EAN-8 ADDENDUM NO 5 Characters 2 Characters 2 or 5 Characters

FORCE UPC-E TO UPC-A FORMAT

Yes No

34

FORCE UPC-A TO EAN-13 FORMAT

Yes No

TRANSMIT UPC-A LEADING CHARACTER

Yes No

TRANSMIT UPC-A CHECK DIGIT

Yes No

TRANSMIT UPC-E LEADING CHARACTER

Yes No

|  | End of Configuration |
| --- | --- |

35

| Start of Configuration |  |
| --- | --- |

TRANSMIT UPC-E CHECK DIGIT

Yes No

TRANSMIT EAN-13 CHECK DIGIT

Yes No

TRANSMIT EAN-8 CHECK DIGIT

Yes No

36

### CODABAR/ MONARCH PARAMETERS SETTING

START/ STOP CHARACTER TRANSMISSION

No A, B, C, D DC1~DC4 a/ t, b/ n, c/ *, d/ e

CONCATENATION

Enable Disable

### CODE 128 PARAMETERS SETTING

FNC 2 CONCATENATION

Enable Disable

|  | End of Configuration |
| --- | --- |

37

| Start of Configuration |  |
| --- | --- |

CODE 128 PARAMETERS SETTING (Cont’d)

CHECK DIGIT

No Calculate but not Transmit Calculate and Transmit

UCC/EAN128 PARAMETERS SETTING

The character FNC1 can be transmitted or not using these codes.

FNC1 Character Transmitted FNC1 not Transmitted

MATRIX 25 PARAMETERS SETTING

#### Examples: Felting length 4 to 8 characters

| MAX MIN Set | Scan. Start of configuration Min 0 4 set | max 0 8 set End of configuration |
| --- | --- | --- |

38

MSI/PLESSY PARAMETERS SETTING

Examples: Felting length 4 to 8 characters

| MAX MIN SET | Scan:. Start of configuration Min 0 4 set | max 0 8 set End of configuration |
| --- | --- | --- |

Double Check digit

| Calculate but not Transmitted No Calculate but only first one Transmitted Calculated and both Transmitted Single Check digit Calculated but not Transmitted Calculated and transmitted |  |
| --- | --- |
|  | End of Configuration |

39

| Start of Configuration |  |
| --- | --- |

MSI/PLESSY PARAMETERS SETTING (Cont’d)

Plessy code setting Calculated and transmitted Calculate but not transmitted CHECK DIGHT No Calculate and Transmit Calculate but not Transmit

## ITALIAN PHARMACY PARAMETERS SETTING

TRANSMIT "A" CHARACTER

Yes No

40

BARCODE LENGTH SETTING

CODE 39 LENGTH SETTING

MAX MIN

CODE 93 LENGTH SETTING

MAX MIN

CODE 128 LENGTH SETTING

MAX MIN

CODABAR LENGTH SETTING

| MAX MIN SET |  |  |
| --- | --- | --- |
|  | End of Configuration |  |

41

| Start of Configuration |  |
| --- | --- |

ISBN/ ISSN CONVERSION

The function convents the UPC/EAN codes appearing on books and magazine not ISBN/ISSN format.

ACTIVE ISBN/ ISSN INACTIVE ISBN/ ISSN

|  | End of Configuration |
| --- | --- |

42

## DATA EDITING

43

| Start of Configuration |  |
| --- | --- |

## HEADER AND TRAILER

This option allows you to append a header and/or a trailer to every message transmitted via the serial ports or the keyboard port.  There is no restriction in selecting header or trailer characters as far as the sum of the lengths of header and trailer is not greater than 10 digits.

| Header Trailer Set | - Select either header or trailer you are going to program by scanning the corresponding label - Scan the character(s) you want from the enclosed ASCII table to set as header or trailer (be sure to enable full ASCII code 39 option before you start). - Read the “Set” label to set your choice into memory. |
| --- | --- |

BARCODE IDENTIFIER CODE SELECTION

The series of scanners can transmit max.2-digit barcode identifier code for different types of barcodes.  Use the labels to choose transmit or not transmit predefined barcode identifier code (ID’s are listed on page 2):

Enable Disable

44

## BARCODE IDENTIFIER CODE SETTING

Each of the series type scanners can set max.2 digits as barcode identifier code according to different barcode.  The procedure is as follows:

- Scan “Start of configuration” label

- Scan “Barcode identifier setting code” label.

- Scan the new code mark from ASCII table (max. two digits).  For example, if one “AB” want for code mark then scan “A” and “B”.

- Scan “Set” label.

- Scan “ End of configuration” label.

UPC-E UPC-A EAN-13 EAN-8 Chinese post code ITF 2 OF 5

|  | End of Configuration |
| --- | --- |

45

| Start of Configuration |  |
| --- | --- |

BARCODE IDENTIFIER CODE SETTING (Cont’d)

Coda bar Code 39 Code 128 Code 93 MSI MATRIX 25 Set

46

Truncate Header/Trailer Character

(Version az1.24, dz1.05, ac1.01, dz1.05,pl1.39

Or higher is required)

You can truncate a number header or trailer for a symbology. When you do, the specific character you select is deleted from the symbology you want.

| Truncate header character Truncate trailer character set | 1.scan”start of con- figuration” 2.select”truncate header or truncate trailer 3.scan two barcode value from the full ASCII code table( 0~9) For example, if 2 number header you want clear then scan ”0” and “2” |
| --- | --- |

- Scan” set” barcode

- end of configuration

|  | End of Configuration |
| --- | --- |

47

APPENDIXES

48

APPENDIX A

CODE 39 FULL ASCII CODE TABLE

| ASCII | CODE 39 | VALEUR HEXA. |  | ASCII | CODE 39 | VALEUR HEXA. |
| --- | --- | --- | --- | --- | --- | --- |
| NUL | %U | 00 |  | % | /E | 25 |
| SOH | $A | 01 |  | & | /F | 26 |
| STX | $B | 02 |  | ' | /G | 27 |
| ETX | $C | 03 |  | ( | /H | 28 |
| EOT | $D | 04 |  | ) | /I | 29 |
| ENQ | $E | 05 |  | * | /J | 2A |
| ACK | $F | 06 |  | + | /K | 2B |
| BEL | $G | 07 |  | , | /L | 2C |
| BS | $H | 08 |  | - | - | 2D |
| HT | $I | 09 |  | . | . | 2E |
| LF | $J | 0A |  | / | / | 2F |
| VT | $K | 0B |  | 0 | 0 | 30 |
| FF | $L | 0C |  | 1 | 1 | 31 |
| CR | $M | 0D |  | 2 | 2 | 32 |
| SO | $N | 0E |  | 3 | 3 | 33 |
| SI | $O | 0F |  | 4 | 4 | 34 |
| DLE | $P | 10 |  | 5 | 5 | 35 |
| DC1 | $Q | 11 |  | 6 | 6 | 36 |
| DC2 | $R | 12 |  | 7 | 7 | 37 |
| DC3 | $S | 13 |  | 8 | 8 | 38 |
| DC4 | $T | 14 |  | 9 | 9 | 39 |
| NAK | $U | 15 |  | : | /Z | 3A |
| SYN | $V | 16 |  | ; | %F | 3B |
| ETB | $W | 17 |  | < | %G | 3C |
| CAN | $X | 18 |  | = | %H | 3D |
| EM | $Y | 19 |  | > | %I | 3E |
| SUB | $Z | 1A |  | ? | %J | 3F |
| ESC | %A | 1B |  | @ | %V | 40 |
| FS | %B | 1C |  | A | A | 41 |
| GS | %C | 1D |  | B | B | 42 |
| RS | %D | 1E |  | C | C | 43 |
| US | %E | 1F |  | D | D | 44 |
| SP | SP | 20 |  | E | E | 45 |
| ! | /A | 21 |  | F | F | 46 |
| " | /B | 22 |  | G | G | 47 |
| # | /C | 23 |  | H | H | 48 |
| $ | /D | 24 |  | I | I | 49 |

49

APPENDIX A

CODE 39 FULL ASCII CODE TABLE

| ASCII | CODE 39 | VALEUR HEXA. |  | ASCII | CODE 39 | VALEUR HEXA. |
| --- | --- | --- | --- | --- | --- | --- |
| J | J | 4A |  | e | +E | 65 |
| K | K | 4B |  | f | +F | 66 |
| L | L | 4C |  | g | +G | 67 |
| M | M | 4D |  | h | +H | 68 |
| N | N | 4E |  | i | +I | 69 |
| O | O | 4F |  | j | +J | 6A |
| P | P | 50 |  | k | +K | 6B |
| Q | Q | 51 |  | l | +L | 6C |
| R | R | 52 |  | m | +M | 6D |
| S | S | 53 |  | n | +N | 6E |
| T | T | 54 |  | o | +O | 6F |
| U | U | 55 |  | p | +P | 70 |
| V | V | 56 |  | q | +Q | 71 |
| W | W | 57 |  | r | +R | 72 |
| X | X | 58 |  | s | +S | 73 |
| Y | Y | 59 |  | t | +T | 74 |
| Z | Z | 5A |  | u | +U | 75 |
| [ | %K | 5B |  | v | +V | 76 |
| \ | %L | 5C |  | w | +W | 77 |
| ] | %M | 5D |  | x | +X | 78 |
| ^ | %N | 5E |  | y | +Y | 79 |
| _ | %O | 5F |  | z | +Z | 7A |
| ` | %W | 60 |  | { | %P | 7B |
| a | +A | 61 |  | ¦ | %Q | 7C |
| b | +B | 62 |  | } | %R | 7D |
| c | +C | 63 |  | ~ | %S | 7E |
| d | +D | 64 |  | DEL | %T | 7F |

50

APPENDIX A

##### FUNCTION KEY EMULATION

| FUNCTION KEY | ASCII | CODE 39 |  | FUNCTION KEY | ASCII | CODE 39 |
| --- | --- | --- | --- | --- | --- | --- |
| Ins | $A | 01 |  | F1 | $Q | 11 |
| Del | $B | 02 |  | F2 | $R | 12 |
| Home | $C | 03 |  | F3 | $S | 13 |
| End | $D | 04 |  | F4 | $T | 14 |
| Up | $E | 05 |  | F5 | $U | 15 |
| Down | $F | 06 |  | F6 | $V | 16 |
| Left | $G | 07 |  | F7 | $W | 17 |
| Backspace | $H | 08 |  | F8 | $X | 18 |
| TAB | $I | 09 |  | F9 | $Y | 19 |
| Enter(num) | $J | 0A |  | F10 | $Z | 1A |
| Right | $K | 0B |  | F11 | %A | 1B |
| PgUp | $L | 0C |  | F12 | %B | 1C |
| Enter | $M | 0D |  | ESC | %C | 1D |
| PgDn | $N | 0E |  | Ctl(L) | %D | 1E |
| shift | $O | 0F |  | Alt(L) | %E | 1F |
| 5 (num) | $P | 10 |  |  |  |  |

51

APPENDIX B

CODE 39 FULL ASCII BARCODE TABLE

Start of Configuration

| NUL | ENQ (Up) |
| --- | --- |
| SOH (Ins) | ACK (Down) |
| STX (Del) | BEL (Left) |
| ETX (Home) | BS (Backspace) |
| EOT (End) | HT (TAB) |

52

APPENDIX B

CODE 39 FULL ASCII BARCODE TABLE

| LF (Enter)(num) | DLE 5 (num) |
| --- | --- |
| VT (Right) | DC1 (F1) |
| FF (PgUp) | DC2 (F2) |
| CR (Enter) | DC3 (F3) |
| SO (PgDn) | DC4 (F4) |
| SI shift(L) | NAK (F5) |

|  | End of Configuration |
| --- | --- |

53

##### APPENDIX B

CODE 39 FULL ASCII BARCODE TABLE

Start of Configuration

| SYN (F6) | GS (ESC) |
| --- | --- |
| ETB (F7) | RS Ctl (L) |
| CAN (F8) | US Alt (L) |
| EM (F9) | SP |
| SUB (F10) | ! |
| ESC (F11) | " |
| FS (F12) | # |

54

APPENDIX B

CODE 39 FULL ASCII BARCODE TABLE

| $ | + |
| --- | --- |
| % | , |
| & | - |
| ' | . |
| ( | / |
| ) | 0 |
| * | 1 |

|  | End of Configuration |
| --- | --- |

55

APPENDIX B

CODE 39 FULL ASCII BARCODE TABLE

Start of Configuration

| 2 | : |
| --- | --- |
| 3 | ; |
| 4 | < |
| 5 | = |
| 6 | > |
| 7 | ? |
| 8 | @ |
| 9 | A |

56

##### APPENDIX B

CODE 39 FULL ASCII BARCODE TABLE

| B | I |
| --- | --- |
| C | J |
| D | K |
| E | L |
| F | M |
| G | N |
| H | O |

|  | End of Configuration |
| --- | --- |

57

APPENDIX B

CODE 39 FULL ASCII BARCODE TABLE

Start of Configuration

| P | W |
| --- | --- |
| Q | X |
| R | Y |
| S | Z |
| T | [ |
| U | \ |
| V | ] |

58

APPENDIX B

CODE 39 FULL ASCII BARCODE TABLE

| ^ | e |
| --- | --- |
| _ | f |
| ` | g |
| a | h |
| b | i |
| c | j |
| d | k |

|  | End of Configuration |
| --- | --- |

59

APPENDIX B

CODE 39 FULL ASCII BARCODE TABLE

Start of Configuration

| l | s |
| --- | --- |
| m | t |
| n | u |
| o | v |
| p | w |
| q | x |
| r | y |

60

APPENDIX B

CODE 39 FULL ASCII BARCODE TABLE

| z | } |
| --- | --- |
| { | ~ |
| ¦ | DEL |

End of Configuration

61

APPENDIX C

BARCODE SAMPLES

Code 39

Code 128

Interleaved 2 of 5

Coda bar(NW-7)

UPC A

EAN-13

62

APPENDIX D

QUICK SETTINGS

1. Quick Settings for Keyboard Wedge Mode

| Program | Reset |
| --- | --- |
| PC/AT | End |

2. Quick Settings for RS 232 Mode

| Program | Reset |
| --- | --- |
| RS-232C | End |

3. Quick Settings for German Language Keyboard

| Program | German Keyboard |
| --- | --- |
| End |  |

63